SECTION 14202

ELEVATORS AND LIFTS

Display hidden notes to specifier. (Don't know how? Click Here)

Copyright 2008 - 2008 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER ** Savaria ; residential and commercial elevators and lifts.
 .
 This section is based on the products of Savaria, which is located at:
2 Walker Drive  
Brampton, ON, Canada L6T 5E1
Toll Free Tel: 800-661-5112
Tel: 905-791-5555
Fax: 905-791-2222
Email: request info
Web: www.savaria.com

As a leading designer and manufacturer of residential elevators, luxury home elevators, commercial elevators and lifts for over 30 years, Savaria Lifts has thousands of satisfied customers to thank for their business.
 Our global operation continuously seeks to advance manufacturing processes in our own modern facilities, allowing us to offer exceptional value and shorter lead times, while never compromising product quality, safety or reliability.
 As an active member of many trade associations, we are committed to the ongoing support of its customers and dealers alike. Authorized dealers are carefully selected for their knowledge and experience, kept up-to-date with the latest technical and training sessions, and supported with our expert customer service.
Commercial Wheelchair Lifts:

V1504: The unique drive and platform combinations of the V-1504 vertical wheel chair platform lift allows for rapid delivery and ease of installation in respect to any architectural requirements.

- GENERAL

  - SECTION INCLUDES

    - Commercial wheelchair lifts.

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant for elevator products; add others as required.

    - Division 16 Sections for electrical service for elevators to and including disconnect and fused switches at machine room.

    - Division 16 Sections for standby power source, transfer switch, and connection from auxiliary contacts in transfer switch to controller.

    - Division 16 Section "Voice and Data Communication Cabling" for telephone service to elevators.

    - Section 03300 - Cast-in-Place Concrete: Concrete for elevator machine foundation, and pit.

    - Section 06100 - Rough Carpentry: Hoistway framing, building-in hoistway door frames and overhead hoist beams.

    - Section 08210 - Wood Doors: Hoistway doors.

    - Section 08710 - Door Hardware.

    - Section 09260 - Gypsum Board Assemblies: Gypsum shaft walls.

    - Section 09650 - Resilient Flooring: Floor finish in cab.

    - Section 09686 - Carpet: Floor finish in cab.

    - Section 09900 - Paints and Coatings: Interior transparent wood finish in cab.

    - Section 13850 - Detection and Alarm: Fire and smoke detectors and interconnecting devices.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not required.

    - American National Standards Institute (ANSI) B-29.2 - Chain Standards for Inverted Tooth (Silent) Chains and Sprockets.

    - American Society of Mechanical Engineers (ASME) A17.1 - Safety Code for Elevators and Escalators.

    - American Society of Mechanical Engineers (ASME) A18.1 - Safety Standard for Platform and Stairway Chair Lifts.

    - CSA B44.1 - Elevator and Escalator Electrical Equipment.

    - CSA B355 - Lifts for Persons with Physical Disabilities.

    - CSA B613 - Private Residence Lifts for Persons with Physical Disabilities.

    - U.S. Architectural & Transportation Barriers Compliance Board's "Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG)".

    - ICC/ANSI A117.1 - Accessible and Usable Buildings and Facilities.

    - NFPA 70 - National Electric Code.

    - CSA - National Electric Code.

  - REQUIREMENTS OF REGULATORY AGENCIES:

    - Fabricate and install work in compliance with applicable jurisdictional authorities.

    - File shop drawings and submissions with local authorities as the information is made available. Company pre-inspection and jurisdictional authority inspections and permits are to be made on timely basis as required.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings: Provide a complete layout of lift equipment detailing dimensions and clearances as required.

** NOTE TO SPECIFIER ** Delete selection samples if colors have already been selected.

    - Selection Samples: For each finish product specified requiring selection of color or finish, two complete sets of color charts representing manufacturer's full range of available colors and patterns.

  - QUALITY ASSURANCE

    - Installer Qualifications:

      - Skilled tradesmen shall be employees of the installing contractor approved by the manufacturer, with demonstrated ability to perform the work on a timely basis.

      - Execute work of this section only by a company that has adequate product liability insurance.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging until ready for installation.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install systems under environmental conditions outside manufacturer's absolute limits.

  - WARRANTY

    - Coverage - this warranty applies to the repair or replacement, at Manufacturer's option, of parts that fail due to defective material or workmanship. Manufacturer may, at its option, provide factory reconditioned parts. This warranty is provided to the Authorized Dealer on behalf of the final purchaser of the product and is not transferable. The Manufacturer's warranty does not cover labor charges for the removal, repair or replacement of warranty parts but such costs may be covered for a period of time by Authorized Dealer's warranty, which is provided to purchaser separately.

      - The manufacturer shall offer a 36-month limited warranty on parts from date of shipment.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:  Savaria, which is located at: 2 Walker Drive, Brampton, ON, Canada, L6T 5E1; Toll Free Tel: 800-661-5112; Tel: 905-791-5555; Fax:  905-791-2222; Email: request info; Web: www.savaria.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

  - COMMERCIAL WHEELCHAIR LIFT

    - Hydraulic Vertical Platform Lifts: Savaria V1504-AL/PE Enclosure:

- Model:

** NOTE TO SPECIFIER ** Please select model type

- V-1504 – Aluminum Enclosure

- V-1504 – Plexiglass Enclosure

- V-1504 – Tempered Glass Enclosure

    - Hydraulic Vertical Platform Lift: The lift described here, manufactured by Savaria Lifts Inc, is a vertical platform lift consisting of a hydraulic tower with a lifting platform. The platform is made to accommodate a wheelchair user or a person with impaired mobility. The lift can be used indoor or outdoor (with optional package) and in commercial or residential applications.

    - Work described in this section includes providing equipment, incidental material and labor required for complete, operable roped hydraulic wheelchair lift installation. Lifts shall be erected, installed, adjusted, tested and placed in operation by lift system manufacturer, or manufacturer's authorized installer.

- Lifts shall be in accordance:

** NOTE TO SPECIFIER ** Select code and accessibility standard required.

- ASME A18.1 and ADAAG compliant (USA)

- ASME A18.1 and A117.1 compliant (USA)

- ASME 18.1 only (USA)

- CSA B355 (Canada)

    - The following preparatory work to receive the lifts specified in this section is part of the work of other sections:

      - Permanent 120 VAC, 20 amp single phase power to operate lift to be provided from a lockable fused/cartridge type disconnect switch with auxiliary contacts for battery operation. Refer to drawings for permanent power specifications and location of disconnects. Temporary power may be provided to expedite installation of lift.

      - Provide rough openings per lift contractor's shop drawings.

      - Provide substantial, level pit floor slab as indicated on the lift contractor's shop drawings.

    - Characteristics:

- Rated Load: 750 lb (340 kg).

- Rated Speed: 20 fpm (0.10 m/s).

- Car Dimensions:

** NOTE TO SPECIFIER ** Delete platform size not required.

To meet ADA on a 90 degree application in the USA platform size “c” or “d” must be selected.

To meet ANSI A117.1 on a 90 degree application in the USA platform size “d” must be selected.

- 36 inches W by 54 inches D (914 mm by 1371 mm) – Standard

- 36 inches W by 48 inches D (914 mm by 1219 mm)

- 36 inches W by 60 inches D (914 mm by 1524 mm)

- 42 inches W by 60 inches D (1067 mm by 1524 mm)

- Levels Serviced:

** NOTE TO SPECIFIER ** Delete levels served not required.

- 2.

- 3.

- 4.

- Car Configuration:

** NOTE TO SPECIFIER ** Delete car configuration not required.

- Enter/Exit same side.

- 90 degree exit.

- Front/rear exit.

** NOTE TO SPECIFIER ** Please enter travel height

- Travel: ______ feet. Maximum of 14’ in US and 23’ in Canada.

- Pit Depth:

** NOTE TO SPECIFIER ** Please select pit requirement

- 3” – Standard

- 0” – Fixed Ramp Required

- Installation Environment

** NOTE TO SPECIFIER ** Please select installation environment

- Indoor (interior install)

- Outdoor (exterior install)

- Powder Coat Finish

** NOTE TO SPECIFIER ** Please select color choice

- Almond beige – Standard

- Optional Color – From manufactures color chart

- Custom Color – Provide color sample to manufacturer

- Operation: Constant pressure.

- Power Supply: 110 volt, 20 amp, 1 phase, 60 Hz.

- Drive System: 2:1 Roller chain hydraulic.

- Emergency Power:

** NOTE TO SPECIFIER ** Please select emergency operation requirement

- Battery operation in down direction – Standard.

- 24VDC Battery raising and lowering

- Controller: Relay logic based controller.

- Motor/Pump: 1HP (112 kw), gear type

- Manual Lowering: Outside the hoistway at lower landing.

    - Car Enclosure: Side Guards of platform shall have a steel frame with a powder coat finish and steel panel inserts to a minimum of 42 inches (1067 mm) high.

    - Doors and Gates:

- First landing door:

- Door type: 80” low profile aluminum door with a concealed electro/mechanical interlock.

- Flush closing operation with enclosure side.

- Operation

** NOTE TO SPECIFIER ** Select either a manual or automatic door and delete option not required. Automatic doors should be used on the following platform configurations to meet ADA and ANSI A117.1: 90 Degree, Enter/Exit Same Side and applications with more than 2 stops. Also for all applications with no pit an automatic operator is needed for the lower door.

- Manual – with hydraulic closer

- Automatic - Surface 24 volt door opener with battery back-up for low profile aluminum door.

- Door Width

** NOTE TO SPECIFIER ** Select either a 36” or 42” wide door and delete the option not required. A 42” wide door must be used on the 90 degree side. 42” doors can also be used on the narrow side of platforms with a width of 42”.

- 36 inches (889 mm)

- 42 inches (1067 mm)

** NOTE TO SPECIFIER ** Please delete intermediate landing 1 door if no intermediate landing is required.

- Intermediate landing 1 door:

- Door type: 80” low profile aluminum door with a concealed electro/mechanical interlock.

- Flush closing operation with enclosure side.

- Operation

** NOTE TO SPECIFIER ** Select either a manual or automatic door and delete option not required. Automatic doors should be used on the following platform configurations to meet ADA and ANSI A117.1: 90 Degree, Enter/Exit Same Side and applications with more than 2 stops.

- Manual – with hydraulic closer

- Automatic - Surface 24 volt door opener with battery back-up for low profile aluminum door.

- Door Width

** NOTE TO SPECIFIER ** Select either a 36” or 42” wide door and delete the option not required. A 42” wide door must be used on the 90 degree side. 42” doors can also be used on the narrow side of platforms with a width of 42”.

- 36 inches (889 mm)

- 42 inches (1067 mm)

** NOTE TO SPECIFIER ** Please delete intermediate landing 2 door if no intermediate landing is required.

- Intermediate landing 2 door:

- Door type: 80” low profile aluminum door with a concealed electro/mechanical interlock.

- Flush closing operation with enclosure side.

- Operation

** NOTE TO SPECIFIER ** Select either a manual or automatic door and delete option not required. Automatic doors should be used on the following platform configurations to meet ADA and ANSI A117.1: 90 Degree, Enter/Exit Same Side and applications with more than 2 stops.

- Manual – with hydraulic closer

- Automatic - Surface 24 volt door opener with battery back-up for low profile aluminum door.

- Door Width

** NOTE TO SPECIFIER ** Select either a 36” or 42” wide door and delete the option not required. A 42” wide door must be used on the 90 degree side. 42” doors can also be used on the narrow side of platforms with a width of 42”.

- 36 inches (889 mm)

- 42 inches (1067 mm)

- Upper landing door/gate:

- Door/gate type:

** NOTE TO SPECIFIER ** Please select door/gate type and remove option not required.

- 80” low profile aluminum door with a concealed electro/mechanical interlock.

- 42” low profile aluminum gate with a concealed electro/mechanical interlock.

- Flush closing operation with enclosure side.

- Operation

** NOTE TO SPECIFIER ** Select either a manual or automatic door/gate and delete option not required. Automatic doors/gates should be used on the following platform configurations to meet ADA and ANSI A117.1: 90 Degree, Enter/Exit Same Side and applications with more than 2 stops.

- Manual – with hydraulic closer

- Automatic - Surface 24 volt door opener with battery back-up for low profile aluminum door.

- Automatic – Surface mounted gate opener for low profile aluminum gate.

- Door/Gate Width

** NOTE TO SPECIFIER ** Select either a 36” or 42” wide door/gate and delete the option not required. A 42” wide door/gate must be used on the 90 degree side. 42” doors can also be used on the narrow side of platforms with a width of 42”.

- 36 inches (889 mm)

- 42 inches (1067 mm)

    - Lift Enclosure:

- The enclosure will be made entirely of aluminum for durability against corrosion.

- The enclosure frames and panels shall be fully assembled and screwed together from inside the enclosure for ease of assembling and quick installation time.

- All enclosure inserts shall be replaceable from the inside of the enclosure for ease of service.

- Fully enclosed unit

** NOTE TO SPECIFIER ** Please select whether unit will be fully enclosed. If required the upper landing must have an 80” low profile aluminum door.

- Yes – Lift will be fitted with ventilation & dome roof to protect from the outdoor elements.

- No – Lift will be open at upper landing.

    - Call Stations: Provide flush, surface or door frame mounted landing call/send stations.

- Call stations will be:

** NOTE TO SPECIFIER ** Select required key switch option below and delete items not required.

- Keyless

- Keyed (removable in off position)

- Keyed (removable in on/off position)

    - Car Operation:

      - Car Operating Panel shall consist of constant pressure buttons, emergency stop/alarm button, on/off key switch (when applicable) and emergency LED light mounted on a removable stainless steel panel (Type 304 #4 Stainless Steel Finish).

      - Auxiliary lighting: The car shall be equipped with a battery operated LED light fixture. The battery shall be the rechargeable type with an automatic recharging system.

      - Telephone: The car shall be equipped with a

** NOTE TO SPECIFIER ** Select required phone option below and delete item not required.

- Hand held recessed telephone

- ADA Hands free phone.

    - Pumping Unit and Control:

      - The pumping unit and control shall be enclosed in the tower. The controller and pump unit shall be pre-wired and tested prior to shipment. The controller is to be relay logic based operation for ease of maintenance and service. Pump unit shall incorporate the following features:

      - Adjustable pressure relief valve.

      - Manually operable down valve to lower lift in the event of an emergency. This valve shall be activated from outside of the hoistway through a keyed box.

      - Pressure gauge isolating valve, manually operable.

      - Gate valve to isolate cylinder from pump unit.

      - Electrical solenoid for down direction control.

      - Emergency Operation - A manual lowering device shall be located outside the hoistway in a lockable box positioned at a lower landing.

    - Cylinder And Plunger:

      - The cylinder shall be constructed of steel pipe of sufficient thickness and suitable safety margin. The top of the cylinder shall be equipped with a cylinder head with an internal guide ring and self-adjusting packing.

      - The plunger shall be constructed of a steel shaft of proper diameter machined true and smooth. The plunger shall be provided with a stop electrically welded to the bottom to prevent the plunger from leaving the cylinder.

    - Roller Chains: Two No.50 roller chains with 5/8 inch (16 mm) pitch. Minimum breaking strength 6100 lb (2773 kg) each.

    - Leveling Device:

      - The lift shall be provided with an anti-creep device which will maintain the carriage level within 1/2 inch (12 mm) of each landing.

      - All limit switch and leveling device switches shall be located in a position to be inaccessible to unauthorized persons. They shall be located behind the mast wall and be accessible through removable panels.

    - Guide Yoke: The 2:1 guide yoke/sprocket assembly shall be supplied with idler sheaves, roller guide shoes, bearings and guards.

    - Terminal Stopping Devices: Normal terminal stopping devices shall be provided at top and bottom of runway to stop the car positively and automatically.

    - Guide Rails and Brackets: Steel 'C'' guide rails and brackets shall be used to guide the platform and sling. Guide rails shall form part of the structural integrity of the unit and be integral to the mast enclosure, ensuring stability and minimum platform deflection when loaded.

    - Car Sling: Car sling shall be fabricated from steel tubing 44 inches (1116 mm) high with adequate bracing to support the platform and car enclosure. Roller guide shoes shall be mounted on the top and bottom of the car sling to engage the guide rails. Guide shoes shall be roller type with 3 inches (76 mm) diameter wheels. Nylon guide shoes shall not be used for better ride quality and durability.

    - Wiring: All wiring and electrical connections shall comply with applicable codes. Insulated wiring shall have flame-retardant and moisture-proof outer covering and shall be run in conduit or electrical wire ways if located outside the unit enclosure. Quick disconnect harnesses shall be used when possible.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until hoistway and machine room has been properly prepared.

    - Site dimensions shall be taken to verify that tolerances and clearances have been maintained and meet local regulations.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

** NOTE TO SPECIFIER ** Delete if not required.

  - ELEVATOR INSTALLATION

    - Install in accordance with manufacturer's instructions.

    - Install the components of the elevator system that are required and that are required by jurisdictional authorities to license the elevator.

    - Trained employees of the elevator contractor shall perform installation work.

    - Adjust elevator for proper operation and clean unit thoroughly.

    - Instruct users in operating procedures and owner's maintenance person in trouble-shooting and maintenance procedures.

** NOTE TO SPECIFIER ** Delete if not required.

  - LIFT INSTALLATION

    - Install all the components of the lift system that are specified in this section to be provided, and that are required by jurisdictional authorities to license the lift.

    - Trained employees of the lift contractor shall perform all installation work of this section.

    - Adjust lift for proper operation and clean unit thoroughly.

    - Instruct users in operation procedures and Owner's maintenance person in trouble-shooting and maintenance procedures.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION